Experiment 4

Verification of System Properties

Objective 1: Linearity and Time Invariance

To verify the linearity and time-invariance properties of continuous-time and discrete-time systems.

CODE :

% Experiment 11: Linearity and Time Invariance

clc; clear; close all;

t = -1:0.001:1;

x1 = sin(2*pi*5*t);

x2 = cos(2*pi*5*t);

a = 2;

b = 3;

t0 = 0.2;

T = @(x) 2*x;

lhs = T(a*x1+b*x2);

rhs = a*T(x1)+b*T(x2);

linError = max(abs(lhs-rhs));

xDelay = sin(2*pi*5*(t-t0));

lhsTI = T(xDelay);

rhsTI = T(x1);

rhsTIshift = interp1(t,rhsTI,t-t0,'linear',0);

tiError = max(abs(lhsTI-rhsTIshift));

figure;

subplot(2,1,1);

plot(t,lhs-rhs);

grid on;

title('Linearity Error');

subplot(2,1,2);

plot(t,lhsTI-rhsTIshift);

grid on;

title('Time-Invariance Error');

fprintf('Linearity error = %g\n',linError);

fprintf('Time-invariance error = %g\n',tiError);

Objective 2: Causality and Non-Causality

Aim: To classify systems as causal or non-causal based on their input-output relationships.

CODE :

% Experiment 12: Causality and Non-Causality

clc; clear; close all;

n = -10:10;

x = sin(0.2*pi*n);

yC = x + [0 x(1:end-1)];

yNC = x + [x(2:end) 0];

figure;

subplot(3,1,1);

stem(n,x,'filled');

grid on;

title('Input x[n]');

subplot(3,1,2);

stem(n,yC,'filled');

grid on;

title('Causal System y[n]=x[n]+x[n-1]');

subplot(3,1,3);

stem(n,yNC,'filled');

grid on;

title('Non-Causal System y[n]=x[n]+x[n+1]');

Objective 3: Memoryless and Dynamic Systems

Aim: To classify systems based on whether their outputs depend only on the present input or also on other input values.

CODE :

% Experiment 13: Memoryless and Dynamic Systems

clc; clear; close all;

n = -10:10;

x = sin(0.2*pi*n);

yMemoryless = 3*x;

yDynamic = x + [0 x(1:end-1)];

figure;

subplot(3,1,1);

stem(n,x,'filled');

grid on;

title('Input x[n]');

subplot(3,1,2);

stem(n,yMemoryless,'filled');

grid on;

title('Memoryless: y[n]=3x[n]');

subplot(3,1,3);

stem(n,yDynamic,'filled');

grid on;

title('Dynamic: y[n]=x[n]+x[n-1]');

Objective 4: BIBO Stability

Aim: To verify the stability of systems and classify them as stable or unstable.

CODE :

% Experiment 14: BIBO Stability

clc; clear; close all;

t = 0:0.001:5;

x = sin(2*pi*5*t);

yStable = 0.5*x;

yUnstable = exp(t).*x;

figure;

subplot(3,1,1);

plot(t,x);

grid on;

title('Bounded Input x(t)');

subplot(3,1,2);

plot(t,yStable);

grid on;

title('Stable Output y(t)=0.5x(t)');

subplot(3,1,3);

plot(t,yUnstable);

grid on;

title('Unstable Output y(t)=e^t x(t)');

fprintf('Maximum stable output = %g\n',max(abs(yStable)));

fprintf('Maximum unstable output over simulation = %g\n',max(abs(yUnstable)));