Experiment 3

Energy and Power Analysis of Signals

OBJECTIVE:

To calculate the energy and average power of given continuous-time and discretetime signals and classify them as energy signals, power signals, or neither based on their energy and power characteristics.

CODE :

% Experiment 3: Energy and Power Analysis

clc; clear; close all;

dt = 0.001;

t = 0:dt:5;

xE = exp(-t);

xP = sin(2*pi*5*t);

E_num = trapz(t,abs(xE).^2);

T0 = 1/5;

idx = t <= 2;

P_num = (1/(t(find(idx,1,'last'))-t(1))) * ...

trapz(t(idx),abs(xP(idx)).^2);

figure;

subplot(2,1,1);

plot(t,xE); grid on;

title('Energy Signal e^{-t}u(t)');

xlabel('Time (s)'); ylabel('Amplitude');

subplot(2,1,2);

plot(t,xP); grid on;

title('Power Signal sin(2pi5t)');

xlabel('Time (s)'); ylabel('Amplitude');

fprintf('Numerical energy = %g\n',E_num);

fprintf('Numerical average power = %g\n',P_num);