Experiment 5:

Fourier Series Analysis of Periodic Signals

Objective 1: Fourier Series Representation of Continuous-Time Periodic Signals

CODE :

clc; clear; close all;

figure;

A = 1;

f0 = 1;

T = 1/f0;

t = linspace(0, 2*T, 2000);

% Half-wave rectified sinusoidal signal

x = A*sin(2*pi*f0*t);

x(x < 0) = 0;

N = 10;

a0 = (2/T)*trapz(t, x)/2;

an = zeros(1,N);

bn = zeros(1,N);

for k = 1:N

an(k) = (2/T)*trapz(t, x.*cos(2*pi*k*f0*t));

bn(k) = (2/T)*trapz(t, x.*sin(2*pi*k*f0*t));

end

xFS = a0*ones(size(t));

for k = 1:N

xFS = xFS + an(k)*cos(2*pi*k*f0*t) ...+ bn(k)*sin(2*pi*k*f0*t);end

plot(t,x,'k','LineWidth',1.2);

hold on;

plot(t,xFS,'r--','LineWidth',1.2);

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Fourier Series Representation of a Periodic Signal');

legend('Original Signal','Fourier Series Reconstruction');

Objective 2: Determination of Fourier Coefficients and Harmonic Components

CODE :

clc; clear; close all;

figure;

f0 = 1;

T = 1/f0;

t = linspace(0,T,2000);

x = max(sin(2*pi*f0*t),0);

N = 10;

a0 = (2/T)*trapz(t,x);

an = zeros(1,N);

bn = zeros(1,N);

for k = 1:N

an(k) = (2/T)*trapz(t,x.*cos(2*pi*k*f0*t));

bn(k) = (2/T)*trapz(t,x.*sin(2*pi*k*f0*t));

end

n = 1:N;

C = sqrt(an.^2 + bn.^2);

phase = atan2(-bn,an);

fprintf('DC coefficient a0/2 = %.4f\n',a0/2);

disp(table(n.',an.',bn.',C.',phase.', ...

'VariableNames',{'Harmonic','a_n','b_n','Magnitude','Phase'}));

stem(n,C,'filled');

grid on;

xlabel('Harmonic Number');

ylabel('Harmonic Magnitude');

title('Harmonic Components of Periodic Signal');

Objective 3: Analysis of Amplitude and Phase Spectra

CODE :

clc; clear; close all;

figure;

f0 = 1;

T = 1/f0;

t = linspace(0,T,2000);

x = max(sin(2*pi*f0*t),0);

N = 10;

a0 = (2/T)*trapz(t,x);

an = zeros(1,N);

bn = zeros(1,N);

for k = 1:N

an(k) = (2/T)*trapz(t,x.*cos(2*pi*k*f0*t));

bn(k) = (2/T)*trapz(t,x.*sin(2*pi*k*f0*t));

end

A = sqrt(an.^2 + bn.^2);

P = atan2(-bn,an);

n = 1:N;

subplot(2,1,1);

stem(n,A,'filled');

grid on;

xlabel('Harmonic Number');

ylabel('Amplitude');

title('Amplitude Spectrum');

subplot(2,1,2);

stem(n,P,'filled');

grid on;

xlabel('Harmonic Number');

ylabel('Phase (rad)');

title('Phase Spectrum');

Objective 4: Verification of Fourier Series Reconstruction and Signal Characteristics

CODE :

clc; clear; close all;

figure;

f0 = 1;

T = 1/f0;

t = linspace(0,2*T,4000);

x = max(sin(2*pi*f0*t),0);

N = 10;

a0 = (2/T)*trapz(t(1:2000),x(1:2000));

an = zeros(1,N);

bn = zeros(1,N);

tt = t(1:2000);

xx = x(1:2000);

for k = 1:N

an(k) = (2/T)*trapz(tt,xx.*cos(2*pi*k*f0*tt));

bn(k) = (2/T)*trapz(tt,xx.*sin(2*pi*k*f0*tt));

end

xr = a0/2*ones(size(t));

for k = 1:N

xr = xr + an(k)*cos(2*pi*k*f0*t) ...

+ bn(k)*sin(2*pi*k*f0*t);

end

err = x-xr;

subplot(2,1,1);

plot(t,x,'k',t,xr,'r--','LineWidth',1.1);

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Original and Reconstructed Signals');

legend('Original','Reconstructed');

subplot(2,1,2);

plot(t,err,'b');

grid on;

xlabel('Time (s)');

ylabel('Error');

title('Reconstruction Error');