Experiment 7

Continuous-Time Fourier Transform and Spectrum

Objective 1: Analysis of Continuous-Time Fourier Transform of Different Signals

CODE :

clc; clear; close all;

t = -5:0.001:5;

dt = t(2)-t(1);

signals = {exp(-abs(t)), exp(-t.^2), double(abs(t)<=1)};

names = {'e^{-abs(t)}','Gaussian Pulse','Rectangular Pulse'};

for k = 1:3

x = signals{k};

N = 2^nextpow2(length(x));

X = fftshift(fft(x,N))*dt;

f = (-N/2:N/2-1)/(N*dt);

figure;

subplot(2,1,1);

plot(t,x,'LineWidth',1.1);

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title(['Signal: ',names{k}]);

subplot(2,1,2);

plot(f,abs(X));

grid on;

xlim([-5 5]);

xlabel('Frequency (Hz)');

ylabel('|X(f)|');

title(['Magnitude Spectrum: ',names{k}]);

end

Objective 2: Determination of Magnitude and Phase Spectra

CODE :

clc; clear; close all;

t = -2:0.001:2;

dt = t(2)-t(1);

x = exp(-t.^2).*cos(2*pi*3*t);

N = 4096;

X = fftshift(fft(x,N))*dt;

f = (-N/2:N/2-1)/(N*dt);

magX = abs(X);

phaseX = unwrap(angle(X))

figure;

subplot(3,1,1);

plot(t,x);

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Continuous-Time Signal');

subplot(3,1,2);

plot(f,magX);

xlim([-10 10]);

grid on;

xlabel('Frequency (Hz)');

ylabel('|X(f)|');

title('Magnitude Spectrum');

subplot(3,1,3);

plot(f,phaseX);

xlim([-10 10]);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (rad)');

title('Phase Spectrum');

Objective 3: Verification of Fourier Transform Properties

CODE :

clc; clear; close all;

t = -2:0.001:2;

dt = t(2)-t(1);

x = exp(-t.^2);

x_shift = interp1(t,x,t-0.4,'linear',0);

X = fftshift(fft(x))*dt;

N = length(x);

f = (-N/2:N/2-1)/(N*dt);

% Time-shift property

X_shift_theory = X.*exp(-1j*2*pi*f*0.4);

X_shift = fftshift(fft(x_shift))*dt;

figure;

subplot(2,1,1);

plot(f,abs(X_shift),f,abs(X_shift_theory),'--');

xlim([-5 5]);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Time-Shift Property: Magnitude Comparison');

legend('Computed','Theoretical');

subplot(2,1,2);

plot(f,unwrap(angle(X_shift)), ...

f,unwrap(angle(X_shift_theory)),'--');

xlim([-5 5]);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (rad)');

title('Time-Shift Property: Phase Comparison');

legend('Computed','Theoretical');

Objective 4: Analysis of Signal Bandwidth and Spectral Components

CODE :

clc; clear; close all;

fs = 2000;

t = 0:1/fs:1-1/fs;

% Baseband signal

x = cos(2*pi*50*t) ...

+ 0.6*cos(2*pi*120*t) ...

+ 0.3*cos(2*pi*250*t);

N = length(x);

X = fftshift(fft(x))/N;

f = (-N/2:N/2-1)*(fs/N);

M = abs(X);

figure;

plot(f,M);

grid on;

xlim([-400 400]);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Frequency Spectrum and Spectral Components');

threshold = 0.05*max(M);

idx = find(M >= threshold);

bandwidth = max(abs(f(idx)));

fprintf('Approximate one-sided occupied bandwidth = %.2f Hz\n',bandwidth);