Experiment 8

Laplace Transform Analysis

Objective 1: Analysis of Laplace Transform of Continuous-Time Signals.

Code :

clc;

clear;

close all;

%% 8(i) Laplace Transform Analysis

% Requires Symbolic Math Toolbox

syms t s

x = exp(-2*t)*sin(3*t)*heaviside(t);

X = laplace(x,t,s);

disp('Laplace Transform X(s) =');

pretty(X);

omega = -20:0.05:20;

Xjw = double(subs(X,s,1j*omega));

figure;

plot(omega,abs(Xjw),'LineWidth',1.1);

grid on;

xlabel('\omega (rad/s)');

ylabel('|X(j\omega)|');

title('Magnitude of Laplace Transform on j\omega Axis');

Objective 2: Inverse Laplace Transform and Signal Reconstruction

Code :

clc;

clear;

close all;

%% 8(ii) Inverse Laplace Transform

% Requires Symbolic Math Toolbox

syms s t

X = 1/((s+2)*(s+3));

x = ilaplace(X,s,t);

disp('Inverse Laplace Transform x(t) =');

pretty(x);

tt = 0:0.001:5;

xx = double(subs(x,t,tt));

figure;

plot(tt,xx,'LineWidth',1.2);

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Signal Reconstructed using Inverse Laplace Transform');

Objective 3: Determination of Region of Convergence (ROC)

Code :

clc;

clear;

close all;

%% 8(iii) Region of Convergence (ROC)

poles = [-2 -4];

figure;

plot(real(poles),imag(poles),'rx','MarkerSize',12,...

'LineWidth',2);

hold on;

xline(-2,'--b','Re(s) = -2');

xline(-4,'--g','Re(s) = -4');

grid on;

xlabel('Real Axis');

ylabel('Imaginary Axis');

title('Pole Locations and ROC Boundaries');

legend('Poles','Location','best');

disp('Poles are at s = -2 and s = -4');

disp('Right-sided ROC: Re(s) > -2');

disp('Left-sided ROC: Re(s) < -4');

disp('Two-sided ROC: -4 < Re(s) < -2');

Objective 4: Analysis of System Stability Using Poles, Zeros and ROC

CODE :

clc;

clear;

close all;

%% 8(iv) System Stability Analysis

% Requires Control System Toolbox

num = [1];

den = [1 4 5];

sys = tf(num,den);

disp('Transfer Function:');

disp(sys);

disp('Poles:');

disp(pole(sys));

disp('Zeros:');

disp(zero(sys));

figure;

pzmap(sys);

grid on;

title('Pole-Zero Map');

t = 0:0.01:8;

figure;

impulse(sys,t);

grid on;

title('Impulse Response');

xlabel('Time (s)');

ylabel('h(t)');

figure;

step(sys,t);

grid on;

title('Step Response');

xlabel('Time (s)');

ylabel('y(t)');

p = pole(sys);

if all(real(p) < 0)

disp('The system is BIBO stable.');

else

disp('The system is not asymptotically stable.');

end